CONTAFILL

OPERATING INSTRUCTIONS

The CONTADOR must have the interface option for use with the Contafil. (15 pol.Dsub-plug on the back of the unit.

- Connect the Contafil to the Contador by means of a connection cable to the 15-pol. Plug located on the back of the Contador.   Insert the carousel in the direction indicated by the arrow then turn slightly into the locking position and fix by means of the fastening ring.

- Program the Contador to the “Remote Control” position.

- If there is no container on the Contafil the display will read “CONTAINER”.

- To select the carousel rotation depress the key sequence SET SET 5   .  The function may be changed by depressing the up or down arrow keys.  “1 ROTATION” will fill only one container then stop.  The unit is programmed from the factory at “PRESET 9999” which is continuous filling.  With this setting the operator may replace filled containers for continuous filling.  After the settings have been selected depress “STOP”.

- To rotate the carousel without filling depress the down arrow key one time for each station.

- To begin filling depress “START”.  The carousel will automatically move to the next filling position after filling the present position.

- To interrupt the filling procedure depress “STOP”.  After depressing STOP a new filling procedure will begin, any previously filled positions will not be recognized.

HOFFMAN MANUFACTURING, INC

16541 Green Bridge Road

JEFFERSON, OR 97352 USA

PHONE (541327-2242

FAX (541)327-2255

e-mail: sales@hoffmanmfg.com

www.hoffmanmfg.com