### Oregon Seed Blower

Operating Instructions

Attach the cup housing to the blower cabinet using the screws provided.  Place the cup in the housing.  Be sure the thumbscrews are tight before turning the blower on.  The extension tube supplied with the blower can be used when needed and placed under the upper blower tube.  The principal of the blowing procedure is to remove dead & light seed and particles from the good seed.  After proper completion of the blowing procedure, the good seed will remain in the sample cup and the particles and light seed will be caught in the catch cups or in the tube baffles.

Remove sample cup.  Place sample to be cleaned in the cup and replace cup in blowing position.  For the first separation, set the calibrated airflow control at 10.  After a few separations are performed, the exact settings can be established for each commodity to be cleaned.

Adjust the airflow control until the desired separations are being made.  This can be determined by viewing the seed and particles in the transparent column and catch cups.  After separation is completed and the blower has stopped, remove the sample cup and empty the remaining good sample.  To empty the catch cups, loosen the thumbscrews and twist the cup out.

Hoffman Manufacturing, Inc

## 16541 Green Bridge Road

Jefferson, OR  97352 USA

Phone: 541-327-2242

FAX:    541-327-2255

Oregon Seed Blower

# Assembly Instructions

Assemble cup housing to blower cabinet

using screws provided.  Place cup in the

housing so that upper blower tube can be

placed on top of the cup.  Be sure that the

thumbscrews are tight before turning the

blower on.

Extension tube supplied with the blower

can be used when needed and placed

under the upper blower tube.

Oregon Seed Blower

Limited Warranty

Hoffman Manufacturing, Inc warrants that if your Oregon Seed Blower proves to be defective in workmanship under normal use we will provide, without charge, the parts and labor to remedy any such defect for the period of one year from the date of original purchase.

To obtain service under this warranty you must contact Hoffman Manufacturing, Inc and provide the original serial # and model number.  You may contact Hoffman Manufacturing Inc. by phone 1-800-692-5962 (Int’l. Phone 001-541-926-2920), FAX 1-800-343-6724 (Int’l. FAX 001-541-926-3949), E-mail info@hoffmanmfg.com or mail PO Box 547 Albany, OR USA.

To receive replacement parts under this warranty the original, defective part must be returned to Hoffman Manufacturing, Inc.  Any postage, insurance and shipping costs incurred in presenting or sending for service is your responsibility.  Packing shall be in conformance with the shipping company’s guidelines and insured if required to cover the cost of the part being returned.

This warranty covers only normal consumer use.  Hoffman Manufacturing, Inc. is not responsible for warranty service performed without prior consent of Hoffman Manufacturing, Inc.  This warranty does not cover products which fail to be properly maintained or fail to function properly as a result of misuse, abuse, improper installation, neglect, improper shipping, damage caused by disasters such as fire, flood, lightening, improper electrical current or service provided without the authorization of Hoffman Manufacturing, Inc.

THIS WARRANTY AND REMEDY PROVIDED ARE EXCLUSIVE AND IN LIEU OF ALL OTHER EXPRESS WARRANTIES AND UNLESS STATED HEREIN, ANY STATEMENTS OR REPRESENTATIONS MADE BY ANY OTHER PERSON OR FIRM ARE VOID.  THE DURATION OF ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE SHALL BE LIMITED TO THE DURATION OF THE EXPRESS WARRANTY SET FORTH ABOVE.  EXCEPT AS PROVIDED IN THIS WRITTEN WARRANTY, NEITHER HOFFMAN MANUFACTURING, INC. NOR ITS AFFILIATES SHALL BE LIABLE FOR ANY LOSS, INCONVENIENCE, OR DAMAGE, INCLUDING DIRECT, SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES, RESULTING FROM THE USE OR INABILITY TO USE THE HOFFMAN PRODUCT, WHETHER RESULTING FROM BREACH OF WARRANTY OR ANY OTHER LEGAL THEORY.

Some states do not allow limitations on how long an implied warranty lasts and some states do not allow the exclusion or limitation of incidental or consequential damages, so the above limitation and exclusion may not apply to you.

This warranty gives you specific legal rights, and you may also have other rights, which vary from state to state.

Hoffman Manufacturing, Inc

## 16541 Green Bridge Road

Jefferson, OR  97352 USA

Phone: 541-327-2242

FAX:    541-327-2255